Notice of Development of Rulemaking

WATER MANAGEMENT DISTRICTS

Suwannee River Water Management District

RULE NOS.:	RULE TITLES:

40B-3.037	Water Well Contractor Licensing

40B-3.504	Location

40B-3.517	Grouting and Sealing

40B-3.902	Forms and Instructions

PURPOSE AND EFFECT: The Suwannee River Water Management District (District) gives notice that it is initiating rulemaking for the purpose of amending water well rules to revise citations to Chapters 62-531, F.A.C. and 62-532, F.A.C., eliminate grouting and sealing rules that are either duplicative of rule 62-532.500, F.A.C., or that do not provide suitable alternatives as provided in subparagraph 62-532.500(3)(i)5.a., F.A.C. The effect will be more understandable and streamlined rules; and reduced regulatory burdens.

SUBJECT AREA TO BE ADDRESSED: Well contractor licensing; sanitary hazard setbacks; grouting and sealing of well casings

RULEMAKING AUTHORITY: 373.044, 373.113, 373.171, 373.309 FS.

LAW IMPLEMENTED: 373.046, 373.113, 373.306, 373.308, 373.309, 373.323 FS.

IF REQUESTED IN WRITING AND NOT DEEMED UNNECESSARY BY THE AGENCY HEAD, A RULE DEVELOPMENT WORKSHOP WILL BE NOTICED IN THE NEXT AVAILABLE FLORIDA ADMINISTRATIVE REGISTER.

Pursuant to the provisions of the Americans with Disabilities Act, any person requiring special accommodations to participate in this workshop/meeting is asked to advise the agency at least 5 days before the workshop/meeting by contacting: SRWMD at (386)362-1001 or 1(800)226-1066 (FL only). If you are hearing or speech impaired, please contact the agency using the Florida Relay Service, 1(800)955-8771 (TDD) or 1(800)955-8770 (Voice).

THE PERSON TO BE CONTACTED REGARDING THE PROPOSED RULE DEVELOPMENT AND A COPY OF THE PRELIMINARY DRAFT, IF AVAILABLE, IS: Warren Zwanka, P.G., Suwannee River Water Management District, 9225 CR 49, Live Oak, Florida 32060, (386)362-1001 or 1(800)226-1066 (FL only)

THE PRELIMINARY TEXT OF THE PROPOSED RULE DEVELOPMENT IS:

40B-3.037 Water Well Contractor Licensing.

Minimum standards for licensing water well contractors shall be in accordance with Chapter 62-531, F.A.C., (August 7, 2019), which is incorporated by reference in subsection 40B-3.035(2), F.A.C., and the Water Well Construction Disciplinary Guidelines and Citations Dictionary (June 22, 2014), incorporated by reference in paragraph 40B-3.035(2)(a), F.A.C. Minimum standards for licensing water well contractors shall be in accordance with Chapter 62-531, F.A.C., which also includes the Water Well Contractor Disciplinary Guidelines and Procedures Manual. The licensing program shall be administered and enforced by the District under the authority delegated to it by the Department.

Rulemaking Authority 373.044, 373.113, 373.171 FS. Law Implemented 373.046, 373.308, 373.309, 373.323 FS. History–New 7-1-85, Amended 8-15-89, 6-22-99, [DATE].

40B-3.504 Location.

Wells shall be located so as not to pose a threat of contamination to the groundwater water resource. Minimum spacing between wells and sanitary hazards shall be as set forth in Table 1 of Chapter 62-532, F.A.C., (February 16, 2012), which is incorporated by reference in subsection 40B-3.035(3), F.A.C. and surface waters or sanitary hazards such as septic tanks, drainfields, or cesspools shall be as specified by Section 381.272, F.S., and the respective rules of the Department or DOH. Wells shall not be located on lands below the 100-year flood elevation of the Suwannee River system defined as the 100-year floodplain of the Suwannee, Alapaha, Withlacoochee, Santa Fe, and Aucilla Rivers when an alternative to such location exists on the property.

Rulemaking Authority 373.044, 373.171, 373.309 FS. Law Implemented 17-101.040(12)(a)4., F.A.C.; 373.306, 373.308, 373.309, 373.113 FS. History–New 7-1-85, Amended 4-4-91, 6-22-99, [DATE].

40B-3.517 Grouting and Sealing.

Wells shall be grouted and sealed to protect the groundwater water resource from degradation caused by movement of waters along the well annulus either from the surface to the aquifer or between aquifers, and to prevent loss of artesian pressure in artesian aquifers. All wells shall be constructed and sealed using a method that ensures which insures that an open or unnaturally permeable annular space does not remain when a well is completed. All wells that are constructed in a manner which creates an annular space between the casing and the naturally occurring geologic formations shall be grouted and sealed as set forth in Chapter 62-532, F.A.C., (February 16, 2012), which is incorporated by reference in subsection 40B-3.035(3), F.A.C. will be grouted and sealed in accordance with the methodologies listed in this section.

(1) In order to prevent possible migration of surface waters down the annulus any well located on lands below the 100-year flood elevation of the Suwannee River system defined as the 100-year floodplain of the Suwannee, Alapaha, Withlacoochee, Sante Fe, and Aucilla Rivers shall have a minimum six-inch thickness of grout to a depth of two feet below natural land surface and be in accordance with the methodologies listed in this section.

(2) Wells obtaining water from a consolidated formation and having a casing less than four inches in diameter shall have a nominal one-inch thickness of grout sealing the annular space.

(3) Wells obtaining water from a consolidated formation and having a casing diameter equal to or greater than four inches in diameter shall have a nominal two-inch thickness of grout sealing the annular space.

(4) Where confining beds are present, grout shall extend from the bottom of the casing to the top of or ten (10) feet into the confining bed overlying the producing aquifer, whichever is less.

(5) Where confining beds are not present, grout shall extend from the bottom of the casing to the top of the producing aquifer.

(6) To prevent interchanges of water when multiple aquifers or zones are penetrated, grout shall extend from the bottom of the casing to the top of the uppermost confining bed or ten (10) feet into the uppermost bed, whichever is less.

(1) (7) No change.

(2) (8) Grouting and sealing of well casings not driven or jetted into place shall be accomplished in the following manner unless otherwise authorized by the District in accordance with Section 373.326(1), F.S.: Grouting and sealing of water wells shall be accomplished in the following manner unless otherwise granted a variance by the District in accordance with Section 120.542, F.S.

(a) No change.

(b) The minimum set time for grouting of casing before drilling operations may continue shall will be 12 hours.

(c) No change.

(d) Neat In all cases, neat cement grout shall will be introduced into the annular space from bottom to top.

(9) Other approved methods and materials may be used if deemed by District staff to be reasonable, and assurance is given for resource protection.

(3) (10) In those cases where, during grouting operations, circulation of the neat cement grout is lost so that the annular space being grouted cannot be filled in one continuous operation, a tremie pipe shall be installed in the annular space to a point immediately above the zone of lost circulation; and the annulus shall be filled from that point using clean sand or rock introduced in a manner that prevents bridging within the annular space until a level is reached at which grouting can be continued bridged at that point by sand and/or other suitable material introduced through the pipe until a level is reached at which grouting can be continued.

Rulemaking Authority 373.044, 373.171, 373.309 FS. Law Implemented 17-101.040(12)(a)4., F.A.C.; 373.306, 373.308, 373.309, 373.113 FS. History–New 7-1-85, Amended 4-4-91, 10-16-94, 6-22-99, [DATE].

The following rules are hereby repealed:

40B-3.902 Forms and Instructions.

Rulemaking Authority 373 FS. Law Implemented 120.53, 120.54(14) FS. History–New 6-22-99, Repealed	.