64E-9.017 Enforcement.

Any public pool can be immediately posted closed by the department as not being in compliance with Chapter 64E-9, F.A.C., whenever any of the following conditions occur:

(1)(a) The disinfectant level is below the minimum or above the maximum that is prescribed in subparagraph 64E-9.004(1)(d)2., F.A.C.

(b) The pH of the pool water is below 7.2 or above 7.8.

(c) The clarity of the pool water is such that the main drain grate is not readily visible from the pool deck.

(d) The recirculation system or disinfection feeding equipment is missing or not functioning.

(e) Any other conditions which endangers the health, safety, or welfare of persons using the pool, for example a missing, unsecured or damaged main drain grate. The division or department may attach a sign that states “Pool Closed. This pool is not in compliance with Chapter 64E-9, F.A.C., and may endanger the health, safety or welfare of persons using this facility”. With the department’s permission, the pool operator may remove signs from the pool area immediately following correction of the cited deficiencies provided the county health department is notified of this action at the earliest possible time.

Specific Authority 381.0011, 381.006, 514.021, 514.05 FS. Law Implemented 381.0025, 381.006, 386.01, 386.02, 386.03, 386.041, 386.051, 514.021, 514.04, 514.05, 514.06 FS. History–New 10-5-93, Formerly 10D-5.146, Amended 12-27-98, 5-27-04.