62-18.030 Instrumentation.

Equipment used in making vehicle sound measurement shall meet the following requirements:

(1) Sound Level Meter. Sound level meters shall be of types 1, 2 or S2A meeting the requirements of the American National Standards Institute, Incorporated, or its successor bodies.

(2) Sound Level Calibrator. The sound level calibrator shall calibrate the entire sound level meter with an acoustic calibrator of the coupler type.

(3) Windscreen. A properly installed windscreen recommended by the manufacturer of the sound level meter may be used.

(4) Calibration. Each sound measuring instrument shall be returned to the manufacturer or their authorized service center for calibration within a period of five years. The sound level calibrator shall be returned to the manufacturer or their authorized service center for calibration annually.

Rulemaking Authority 316.293(3), 403.061(7) FS. Law Implemented 316.293, 316.293(3), 403.061, 403.415(5), (9) FS., Chapter 74-110, Laws of Florida. History–New 1-27-74, Amended 7-19-77, Formerly 17-18.03, 17-18.030.