62-18.570 Vehicle Sound Level.

The sound level reading for a new motor vehicle shall be obtained as follows:

(1) Preliminary Runs. Sufficient preliminary runs shall be made to enable the test driver to become familiar with the operation of the vehicle and to stabilize engine operating conditions.

(2) Test Runs. At least four test runs shall be made for each side of the vehicle. When the exhaust outlet is more than 16 feet from the front of the vehicle with a gross weight rating of 10,000 pounds or more, at least two runs shall be made for each side of the vehicle using both the primary and secondary vehicle reference points defined in Section 62-18.510(5) of this part. At least two additional runs shall be made from the reference point that gives the highest readings.

(3) Reported Sound Level. The reported A-weighted sound level for each side of the vehicle shall be the average of the two highest readings on that side which are within 2 dB of each other. The sound level reported for the vehicle shall be the sound level of the loudest side.

Rulemaking Authority 403.061 FS. Law Implemented 403.061, 403.415(5) FS., Chapter 74-110, Laws of Florida. History–New 11-27-74, Formerly 17-18.57, 17-18.570.