- 62-296.409 Sulfur Recovery Plants.

(1) New Plants recovering sulfur from crude oil gas – emissions shall not exceed 0.004 pounds of sulfur dioxide per pound of sulfur input to the recovery system or 0.004 pounds of sulfur dioxide per pound of sulfur removed from an oil well.

(2) Existing Plants (for which a valid Department Construction permit was issued prior to July 1, 1973) recovering sulfur from crude oil gas – emissions shall not exceed 0.08 pounds of sulfur dioxide per pound of sulfur input to the recovery system or 0.08 pounds of sulfur dioxide per pound of sulfur removed from crude oil or gas processed.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for sulfur dioxide shall be EPA Method 6, as described at 40 C.F.R. Part 60, Appendix A-4, adopted and incorporated by reference at Rule 62-204.800, F.A.C. The minimum sample volume shall be 0.71 dry standard cubic feet. Two 20-minute samples (+ or – five minutes) per run required.

(b) Test procedures shall meet all applicable requirements of Chapter 62-297, F.A.C.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.600(9), 17-296.409, Amended 11-23-94, 1-1-96, 7-10-14.