- 62-711.300 Waste Tire Permit Requirements.

(1) General provisions relating to solid waste management may be found in chapter 62-701, F.A.C., including statements of intent, definitions, prohibitions, general permitting requirements, alternate procedures, and forms. Except where the context indicates otherwise, these general provisions apply to this chapter.

(2) Waste tire processing facilities shall obtain a permit and shall meet the requirements for waste tire processing facilities in rule 62-711.530, F.A.C.

(3) Waste tire collection centers which are not exempt form permitting under subsection 62-711.300(9), F.A.C., shall obtain a permit and shall meet the requirements for waste tire collection centers in rule 62-711.550, F.A.C.

(4) Waste tire collectors shall register with the Department and shall meet the requirements for waste tire collectors in rule 62-711.520, F.A.C.

(5) In lieu of obtaining a separate waste tire processing facility permit or collection center permit, solid waste management facilities may submit existing permits for modification to authorize the storage and processing of waste tires if they maintain a waste tire site and process the waste tires for recycling or disposal. Conditions of certification for facilities certified before February 19, 1989, pursuant to chapter 403, part II, F.S., shall be automatically modified pursuant to the provisions of section 403.511(5)(a), F.S. No permit fee shall be required.

(6) Waste tire sites which are not an integral part of a waste tire processing facility shall be closed in compliance with rule 62-711.700, F.A.C.

(7) All permits issued under this rule, except for general permits pursuant to rule 62-711.801, F.A.C., must include an approved closing plan which meets the requirements of rule 62-711.700, F.A.C.

(8) All permit applications shall be accompanied by the appropriate permit fee, as specified in this chapter or in rule 62-4.050, F.A.C.

(9) A permit is not required under this chapter for tire storage at:

(a) A tire retreading business, unless 1,500 or more waste tires are stored on the business premises;

(b) A single facility that, in the ordinary course of business, removes tires from motor vehicles, unless 1,500 or more waste tires are stored on the business premises, or

(c) A retail tire-selling business which is serving as a waste tire collection center, unless 1,500 or more waste tires are stored on the business premises.

(10) Facilities which receive and consume processed tires for use as a fuel source or raw material shall not be required to obtain a waste tire processing facility permit for storage of this material, provided the following specifications and conditions are met:

(a) The processed tires conform to specifications for nominal one-inch chips, as specified in Table I, or conform to specifications for crumb rubber, as specified in Table II.

| TABLE I Nominal 1-inch Processed Tire Chip |  |  |
| --- | --- | --- |
| Characteristic | Specification | Testing Procedure |
| Particle Size | ≤ 10% by weight retained on a 2" square sieve | Sieve Analysis Procedure: ASTM D 422-63 (Reapproved 1990) |
|  | ≤ 5% total by weight passing through a # 4 sieve |  |
| Wire Content | ≤ 1% by weight free wire | No established procedure. Magnetically or physically separate and weigh free wire as a % of a sample weighing 15-25 pounds |
|  | ≤ 3% of the particles contain bead wire | No established procedure. Physically separate and count particles containing bead wire as a % of a sample of at least 100 particles |

| TABLE II Crumb Rubber |  |  |
| --- | --- | --- |
| Characteristic | Specification | Testing Procedure |
| Particle Size | 0% by weight retained on a .05" square sieve | Sieve Analysis Procedure: ASTM D 422-63 (Reapproved 1990) |
| Wire Content | ≤ 1% by weight free wire | No established procedure. Magnetically or physically separate and weigh free wire as a % of total sample weighing 15-25 pounds |
| Fabric Content | ≤ 10% by weight | No established procedure. Physically separate and weigh fabric as % of a total sample weighing 15-25 pounds |

(b) Storage of either nominal one-inch chips or crumb rubber shall conform to paragraphs 62-711.540(1)(b), (g) and (h), subsections (2) and (3), F.A.C., except that maximum pile height shall be limited to 10 feet.

(c) For facilities consuming nominal one-inch chips, the maximum processed tire inventory shall be limited to the following:

1. For facilities which have been in operation less than six months, one month’s projected usage based on design capacity, or

2. For other facilities, two times the average actual monthly usage during the preceding six months.

3. For single project applications, the entire stockpile shall be consumed within 120 days of its initial formation.

(d) Processed tires, including those meeting the above specifications for nominal one-inch chips or crumb rubber, shall be transported only by a registered waste tire collector.

(e) Facilities which cut, shred, or otherwise alter whole waste tires, or accept processed tires for further processing, are considered waste tire processing facilities, and the processed tires produced will be considered waste tires, even if the resulting processed tire material meets the above specifications for nominal one-inch chips or crumb rubber. Facilities that receive, consume, or process whole waste tires or processed tires other than these specified types and sizes will continue to be regulated as waste tire processing facilities.

(f) Specific references to the following document is made in this rule: ASTM Method D 422-63 (Reapproved 1990), Standard Test Method for Particle-Size Analysis of Soils, Reapproved and editorial changes made September 1990. This document is adopted as a standard and is incorporated into this rule by reference. The reference document is available for inspection at any of the Department’s district offices.

Rulemaking Authority 403.704, 403.717 FS. Law Implemented 403.707, 403.717 FS. History–New 2-19-89, Amended 11-7-90, 2-28-94, Formerly 17-711.300, Amended 12-17-96, 3-22-00.